# AUTHENTICATOR THE ADMINISTRATOR PASSWORD

# PRESS * (OFF GREEN LITE)

# PRESS #

# ENTER DEFAULT PASSWORD: 1234 (ON GREEN LITE)

# PRESS * (EXIT)

# CHANGE ADMINISTRATOR PASSWORD

# THE ADMINISTATOR PASSWORD AUTHENTICATION

# PRESS 8

# ENTER NEW PASSWORD

# ENTER CONFORM NEW PASSWORD

# SUCCEDED (ON GREEN LITE & LONG BEEP)

# EXIT

# ADD USERS

# THE ADMINISTATOR PASSWORD AUTHENTICATION

# PRESS 1

# REGISTER FINGERPRINT 3 TIMES

# SUCCESSFUL

# PRESS * (EXIT)

# REGISTER CARD

# THE ADMINISTATOR PASSWORD AUTHENTICATION

# PRESS 6

# SWIPE CARD

# CARDS ARE SUCCESSFUL

# PRESS * (EXIT)

# DELETE USERS

# THE ADMINISTATOR PASSWORD AUTHENTICATION

# PRESS 2

# DELET USER SUCCESSFUL

# PRESS REGISTER FINGERPRINT OR CARD

# PRESS * (EXIT)

# DELETE ALL USERS

# THE ADMINISTATOR PASSWORD AUTHENTICATION

# PRESS 9

# DELETING ALL USERS SUCCESSFUL

# PRESS * (EXIT)

# CONFIGURE UNLOCK DURATION

# THE ADMINISTATOR PASSWORD AUTHENTICATION

# PRESS 4

# ENTER UNLOCKING DURATION 0 TO 255 (EX. 144)

# THE CONFIGURATION SUCCESSFUL

# PRESS * (EXIT)